# MULTIMEDIA UNIVERSITY OF KENYA

## DEPARTMENT OF ELECTRICAL & TELECOMMUNICATIONS ENGINEERING

# ELECTRICAL ENGINEERING LAB REPORT

## Python Programming for Electrical Engineering Applications

### (Ohm’s Law Calculator ,RC Circuit Simulation ,Data Logging & Plotting)

## STUDENT DETAILS

Name: Donald Kiprop
Registration Number: ENG-219-108/2025
Course Unit: fundamentals of computing
Submited to-----------------
Date of Submission: ____ / ____ / 2026

## TABLE OF CONTENTS

- Lab 1: Ohms Law Calculator in Python
1.1 Title
1.2  Objectives
1.3 Introduction/Theory
1. 4 Procedure
1.5 Results
1.6 D iscussion
1.7 Sources of Errors
1.8 Conclusions
1.9 Referens

- Lab 2: RC Circuit Simulation (Charging & Discharging)
2.1 Title
2.2 Objectives
2.3 Introduction /Theory
2.4 Procedure
2.5 Results
2.6 Discussion
2.7 Sources of Error
2.8 Conclusion
2.9 References

- Lab 3: Temperature Data Logging & Plotting
3.1 Title
3.2 Objectives
3.3 Introduction/Theor
3.4 Procedre
3.5 Results
3.6 Discussion
3.7 Sources f Error
3.8 Cnclusion
3.9 Refereces

# ✅ LAB 1: OHM’S LAW CALCULATOR IN PYTHON

## 1. Title

Ohms Law Calculator Using coding for Current and Power Computation

## 2. Objectives

The objectives of this lab are:

To  write a  program thatcalculates   current using  Ohm’s Law

- To modify the program to allow multiple calculations

- To add exception handling for invalid inputs

- To extend the program to compute power using (P = V \times I

## 3. Introduction / Theory

Ohm’s  Law is a  fundamental law in electrical engineering that explains the relationship between voltage, current,   and resistance  in a  circuit.

### Ohm’s Law

I=[v] *r]

Where:

- (I) = Current (A)

- (V) = Voltage (V)

- (R) = Resistance (Ω)

### Power Equation

Power is the rate of energy transfer in an electrical circuit per init time

P = V \times 

Where:

(P) = Power (W)

This lab demonstrates how we can automate calculations used in a given  circuit analysis and electrical de

## 4. Procedure

- Open VS Code

- Entered voltage (V) and resistance (R) as user inputs

- Computed current using (I = V/R)

- Added a loop to allow multiple calculations

- Included error handling to avoid

  - division by zero

  - non-nueric inputs

- Extended the program to compute power using (P = V \times

- Tested the program with several values.

## 5. Results

Sample test values and outputs:

| Voltage (V) | Resistance (Ω) | Current (A) | Power (W) |
| --- | --- | --- | --- |
| 10 | 5 | 2.000 | 20.000 |
| 12 | 6 | 2.000 | 24.000 |
| 9 | 3 | 3.000 | 27.000 |

### Sample Output Screenshot (Insert if required)

📌 Figure 1:to be added

## 6. Discussion

The results obtained agre with Ohm’s Law  Increasing voltage increases current while increasing resistance decreases current. The program was improved by adding a loop and exception handling which made it more userfriendly and reliable

Power calculations also showed that higher current results in higher power consumption which is important in selecting proper circuit ratings to avoid overheating and component failure

## 7. Sources of Errors

- User typing wrong values during input.

- Real-world circuits have some tolerance (resistors are not exact)

## 8. Conclusion

This lab successfully implemented a code for Ohm’s Law calculations The program correctly computed current and power, supported repeated calculations, and handled invalid input errors. The lab demonstrated the usefulness of programming in solving basic electrical engineering computations efficiently

## 9. References (IEEE)

none

## ✅ Lab 1 Code (Final Solution)

Ohm's Law Calculator with Loop + Error Handling + Power Calculation

while True:

try:

voltage = float(input("Enter voltage (V): "))

resistance = float(input("Enter resistance (Ohms): "))

if resistance == 0:

print("Error: Resistance cannot be zero.\n")

continue

current = voltage / resistance

power = voltage * current

print(f"Current (I) = {current:.3f} A")

print(f"Power (P) = {power:.3f} W\n")

except ValueError:

print("Error: Please enter valid numeric values.\n")

choice = input("Do you want to calculate again? (yes/no): ").lower()

if choice != "yes":

print("Program ended.")

break

# ✅ LAB 2: RC CIRCUIT SIMULATION (CHARGING & DISCHARGING)

## 1. Title

Simulation of Charging and Discharging of a Capacitor in an RC Circuit Using code

## 2. Objectives

- To simulate capacitor charging in an RC circuit using code

- To modify the program to simulate capacitor discharging

- To change (R) and (C) values to observe the time constant effect.

- To relate the simulation to realworld RC circuits.

## 3. Introduction / Theory

An RC circuit contains a resistor (R) and a capacitor (C). When a DC voltage is applied, the capacitor charges gradually and approaches the supply voltage.

### Time Constant

t = RC

This value determines the speed of charging/discharging.

### Charging Equation

Vc(t)=v*e^-t(RC)

### Discharging Equation

Vc(t)=v*e^-t(RC)

At (t = RC) the capacitor reaches about 63.2% of its final charging voltage.

## 4. Procedure

- Opened VScode

- Defined circuit parameters (R), (C), and (V)

- Computed the time constant (T = RC)

- Generated a time array for simulation.

- Calculated capacitor voltage for charging.

- Calculated capacitor voltage for discharging.

- Plotted both curves on one graph.

- Changed values of (R) and (C) and observed the curve changes.

## 5. Results

Using:

- (R = 1000\Omega)

- (C = 1\mu F = 1 \times 10^{-6}F)

- (V = 5V)

[
\tau = RC = 1000 \times 10^{-6} = 0.001s
]

### Graph Output

📌 Figure 2: RC charging and discharging curves (Voltage vs

To be added

## 6. Discussion

The results followed the theoretical exponel behavior of RC circuits, The capacitor voltage does not rise instantly because the resistor limits current, the time constant  was observed to control the speed of response.

When (R) or (C) was increased the capacitor took longer to charge and discharg. This behavior is useful in real applications such as timing circuits filters, and smoothing circuits.

## 7. Sources of Error

- Simulation assumes ideal capacitor with no leakage.

- Real capacitor has tolerance values.

- Step size (number of points) affects plot smoothness

## 8. Conclusion

This lab successfully simulated capacitor charging and discharging using Python. The simulation results matched the theoretical equations. The time constant (\tau = RC) was shown to control how fast the capacitor voltage changes, demonstrating an important concept in circuit theory.

## 9. References (IEEE)

[1] youtube

## ✅ Lab 2 Code (Final Solution)

import numpy as np

import matplotlib.pyplot as plt

# Parameters

R = 1000          # Ohms

C = 1e-6          # Farads

V = 5             # Volts

tau = R * C

# Time array

t = np.linspace(0, 0.01, 1000)

# Charging and discharging equations

Vc_charge = V * (1 - np.exp(-t / tau))

Vc_discharge = V * np.exp(-t / tau)

# Plot

plt.plot(t, Vc_charge, label="Charging")

plt.plot(t, Vc_discharge, label="Discharging")

plt.xlabel("Time (s)")

plt.ylabel("Capacitor Voltage Vc (V)")

plt.title("RC Circuit: Charging and Discharging")

plt.legend()

plt.grid(True)

plt.show()

# ✅ LAB 3: TEMPERATURE DATA LOGGING AND PLOTTING

## 1. Title

Temperature and Humidity Data Lagging and Plotting Using Python

## 2. Objectives

- To log temperature data into a file using a program

- To plot temperature readings using

- To modify the program to include humidity logging.

- To save data in  format.

- To implement error handling in file operations.

## 3. Introduction / Theory

Data logging is important in engineering because it allows continuous monitoring and analysis of system behavior, Logdata can be used to detect faults, track performance, and make decisions.

Python provides file handling and visualization tools that simplify data recording and analysis.

## 4. Procedure

- Imported libraries: random, time, csv, and matplotlib.

- Created lists to store temperature and humidity readings.

- Generated sample sensor values using random variation.

- Opened a CSV file and wrote headers.

- Logged readings in a loop.

- Plotted temperature graph.

- Plotted humidity graph.

- Added exception handling to prevent file errors.

## 5. Results

A file named sensor_log.csv was created with temperature and humidity readings.

### Graph Output

📌 Figure 3: Temperature Log Graph (Temperature vs Sample)
📌 Figure 4: Humidity Log Graph (Humidity vs Sample

*(to be added fr)

## 6. Discussion

The data varied slightly due to random noise, similar to real sensors. Graphs improved interpretation by clearly showing trends and changes.

In real engineering systems, data logging is used in weather monitoring, industrial automation, and IoT sensor systems.

## 7. Sources of Error

- Values were simulated, not from a real sensor.

- File permission errors could stop logging.

- System delay may slightly affect timing.

## 8. Conclusion

This lab successfully demonstrated logging and plotting of temperature and humidity data using Python. Data was stored in CSV format and visualized using graphs, showing how Python can be applied in real engineering monitoring systems.

## 9. References (IEEE)

[1] Python Software Foundation Python Documentation 2025.
[2] Matplotlib Development Team Matplotlib Documentation 2025.

## ✅ Lab 3 Code (Final Solution)

import random

import time

import csv

import matplotlib.pyplot as plt

temps = []

humidity_values = []

filename = "sensor_log.csv"

try:

with open(filename, mode="w", newline="") as file:

writer = csv.writer(file)

writer.writerow(["Sample", "Temperature(C)", "Humidity(%)"])

for i in range(10):

temp = 25 + random.uniform(-1, 1)

humidity = 60 + random.uniform(-5, 5)

temps.append(temp)

humidity_values.append(humidity)

writer.writerow([i + 1, f"{temp:.2f}", f"{humidity:.2f}"])

time.sleep(0.5)

print(f"Data saved successfully in {filename}")

except Exception as e:

print("Error writing to file:", e)

# Plot temperature

plt.plot(temps)

plt.xlabel("Sample")

plt.ylabel("Temperature (°C)")

plt.title("Temperature Log")

plt.grid(True)

plt.show()

# Plot humidity

plt.plot(humidity_values)

plt.xlabel("Sample")

plt.ylabel("Humidity (%)")

plt.title("Humidity Log")

plt.grid(True)

plt.show()